# Guide: Linux Router (Kea + BIND)

Prerequisites

- Log in as root: sudo -i

- Editor: nano or vi

## 1. Network Configuration (Linux)

Replace X with your PC number.

WAN Interface (eth0)

nmcli con mod eth0 ipv4.addresses 192.168.60.200+X/24
nmcli con mod eth0 ipv4.gateway 192.168.60.254
nmcli con mod eth0 ipv4.dns "192.168.50.165 192.168.50.166"
nmcli con mod eth0 ipv4.method manual
nmcli con up eth0

LAN Interface (eth1)

nmcli con mod eth1 ipv4.addresses 192.168.11.1/24
nmcli con mod eth1 ipv4.method manual
nmcli con up eth1

System Setup

useradd username
passwd username
hostnamectl set-hostname mail.mojesluzba.cz

## 2. Windows Client Setup

- IP Address: 192.168.11.2

- Subnet Mask: 255.255.255.0

- Gateway: 192.168.11.1

- DNS: 192.168.11.1 (Pointing to our Linux BIND server)

## 3. Routing & NAT

Enable IP Forwarding

echo "net.ipv4.ip_forward = 1" > /etc/sysctl.d/ip_forward.conf
sysctl -p /etc/sysctl.d/ip_forward.conf

Enable NAT (Masquerade)

firewall-cmd --zone=public --add-masquerade --permanent
firewall-cmd --reload

Test: Ping 8.8.8.8 from Windows.

## 4. BIND (DNS Server)

Install

dnf install bind bind-utils -y

Configure /etc/named.conf Edit the options { ... }; block. Make sure it looks like this:

options {
    listen-on port 53 { 127.0.0.1; 192.168.11.1; };
    directory       "/var/named";
    allow-query     { localhost; 192.168.11.0/24; };
    forwarders      { 192.168.50.165; };
    recursion yes;
    dnssec-validation no;
};

Start Service

systemctl enable --now named
firewall-cmd --permanent --add-service=dns
firewall-cmd --reload

## 5. Kea (DHCP Server)

Install

dnf install kea -y

Configure /etc/kea/kea-dhcp4.conf Delete everything in the file and paste this exact JSON:

{
"Dhcp4": {
    "interfaces-config": {
        "interfaces": [ "eth1" ]
    },
    "lease-database": {
        "type": "memfile",
        "lfc-interval": 3600
    },
    "subnet4": [
        {
            "subnet": "192.168.11.0/24",
            "pools": [ { "pool": "192.168.11.10 - 192.168.11.100" } ],
            "option-data": [
                {
                    "name": "routers",
                    "data": "192.168.11.1"
                },
                {
                    "name": "domain-name-servers",
                    "data": "192.168.11.1"
                }
            ]
        }
    ]
}
}

Start Service

systemctl enable --now kea-dhcp4
firewall-cmd --permanent --add-service=dhcp
firewall-cmd --reload

## 6. SMTP (Postfix)

Install

dnf install postfix mailx -y

Configure /etc/postfix/main.cf Find and edit these lines:

myhostname = mail.mojesluzba.cz
mydomain = mojesluzba.cz
myorigin = $mydomain
inet_interfaces = all
mynetworks = 127.0.0.0/8, 192.168.11.0/24
home_mailbox = Maildir/

Redirect Root Mail

echo "root: username" >> /etc/aliases
newaliases

Start Service

systemctl enable --now postfix
firewall-cmd --permanent --add-service=smtp
firewall-cmd --reload

Test: echo "Test" | mail -s "Subject" root

## 7. HTTP (Web Server + PHP)

Install

dnf install httpd php -y

Setup Default PHP Page

echo "<?php phpinfo(); ?>" > /var/www/html/index.php

Enable UserDir (/etc/httpd/conf.d/userdir.conf)

- Comment out: #UserDir disabled

- Uncomment: UserDir public_html

Setup User Page

mkdir -p /home/username/public_html
echo "<h1>username Page</h1>" > /home/username/public_html/index.html
chown -R username:username /home/username/public_html
chmod 711 /home/username

SELinux & Start

setsebool -P httpd_enable_homedirs 1
systemctl enable --now httpd
firewall-cmd --permanent --add-service=http
firewall-cmd --reload

## 8. SSH Hardening

Configure /etc/ssh/sshd_config Add or modify:

Port 60555
AllowTcpForwarding no
AllowUsers username

SELinux & Firewall

# Allow port in SELinux
dnf install policycoreutils-python-utils -y
semanage port -a -t ssh_port_t -p tcp 60555

# Allow port in Firewall & Block default SSH
firewall-cmd --permanent --add-port=60555/tcp
firewall-cmd --permanent --remove-service=ssh
firewall-cmd --reload

# Restart SSH
systemctl restart sshd